- PURPOSE

  - The purpose of this SOP is to describe the standard procedure for the transfer of Raw Material in the Sterile Manufacturing Area.

- SCOPE

  - The SOP is applicable to the transfer of Raw Material in the Sterile Manufacturing Area of Liquid Injectable Section.

- RESPONSIBILITY

  - Area Supervisor is responsible for conducting the activities as per SOP.

  - Production Officer is responsible for checking the activities as per SOP.

  - QA Officer is responsible for monitoring the activities as per SOP.

  - Production Manager is responsible to ensure implementation of this SOP.

- MATERIALS & EQUIPMENTS

- Raw Material

- IPA (70% Solution)

- Transfer Hatch

- PROCEDURE

  - The materials which are required in sterile area for manufacturing are sent through transfer hatch.

  - Before transferring the materials through transfer hatch, mop the material container with IPA 70% solution.

  - Transfer the container into transfer hatch using the outside door of hatch and close the door.

  - Switch ON the main power supply and press the start button of transfer hatch. Check that air supply and UV light is ON. Now wait for 5minutes.

  - Open the hatch from sterile side and take the material into the section for use. Close the door.

  - Precautions

- Check that the materials are properly sealed while entering into the area.

- Do not open both doors of the hatch at the same time.

- REPORTING

NA

- REFERENCE DOCUMENTS

- SOP for operation of Transfer Hatch

- SOP for cleaning of Transfer Hatch

- DISTRIBUTION LIST

| DISTRIBUTION | COPY# | COPY HOLDER |
| --- | --- | --- |

- REVISION HISTORY

| REN NO. | DOCUMENT CHANGE REQUEST NO. | DESCRIPTION OF REVISION | APPROVED BY | DATE |
| --- | --- | --- | --- | --- |